PARALAX: LPT Sound System

Universal DB25 Audio Interface for ESP32-TrueVGM-Chip-Player

PARALAX is a cycle-accurate parallel-port sound interface that replaces classic LPT audio devices such as Covox Speech Thing, Disney Sound Source, and OPL2LPT / OPL3LPT.

It captures raw DB25 bus activity using an RP2040 and translates it into modern, deterministic audio and register streams — completely driverless, IRQ-free, and timing-faithful.

PARALAX can operate standalone, or as a high-performance front-end for the
ESP32-TrueVGM-Chip-Player, where it unlocks real Yamaha OPL and PSG hardware playback.

Supported Modes

Covox Speech Thing

- DATA0–7 mapped to 8-bit unsigned PCM.

- STROBE edge latches samples.

- Typical use: speech, samples, digitized effects.

Implementation

- RP2040 PIO captures write edges

- Samples buffered with timestamps

- Streamed as PCM audio

Disney Sound Source

- Parallel FIFO with internal playback clock

- Handshaked write pacing

Implementation

- FIFO behavior emulated in hardware

- Back-pressure preserved

- Accurate playback without TSRs

FTL Sound Adapter

- DAC + timer-based PCM output

- Common in early DOS titles

Implementation

- Timing-aware capture

- Converted to modern audio streams

OPL2LPT / OPL3LPT

- Parallel-port AdLib compatibles

- Register index/data multiplexed via control lines

Implementation

- Register writes captured verbatim

- Forwarded losslessly to:

  - Soft-OPL engine

  - Real YM3812 / YMF262 hardware (via ESP32-TrueVGM Tier-3)

No ISA slots. No emulation guesswork. No reinterpretation.

System Architecture

RP2040 W — The Front-End Interface

- Core 0

  - PIO state machines

  - DB25 protocol decoding

  - Edge timestamping

- Core 1

  - Mode detection

  - Packet framing

  - Optional local monitoring

- I/O

  - DB25 (Parallel Port)

  - USB CDC (Debug / Control)

  - High-speed SPI or I²S (to Tier-3)

The RP2040 handles all timing-critical parallel-port work so the host PC doesn’t have to.

ESP32-TrueVGM Tier-3 — The Audio Engine

When paired with the ESP32-TrueVGM-Chip-Player:

- Daemon Core

  - Watchdog

  - Clock supervision

  - Bus arbitration

- Director Core

  - Auto-detects incoming PARALAX traffic

  - Routes to correct playback engine

- Player Core

  - Soft-OPL emulation or

  - Real hardware bus driver (YM2612, YM3812, YMF262, SN76489)

Visualization and UI reflect actual register activity, not reconstructed waveforms.

Virtual Passthrough (The Lunch-Eater)

Because the RP2040 is USB-native, PARALAX can act as a multi-input audio front-end, not just an LPT decoder.

Supported Operating Modes

- Legacy Mode
DB25 → RP2040 → ESP32 → Speakers
(Pure DOS / retro source)

- Modern Mode
USB Audio → ESP32 → Speakers
(Acts as a standard USB DAC)

- Hybrid Mode
Mix modern OS audio (Discord, Spotify)
with retro LPT audio (Doom, Monkey Island)
in real time, in hardware.

No OS mixers. No latency stacks. No hacks.

Why PARALAX Exists

- Zero Configuration

  - No SOUND.COM

  - No SET BLASTER

  - No IRQ or DMA conflicts

- Universal Compatibility

  - Any PC with a standard DB25 parallel port

  - Real hardware, DOSBox, or virtualized hosts

- Timing-Correct

  - Cycle-accurate edge capture

  - Deterministic playback clocking

- Hardware-Forward

  - Native support for real Yamaha OPL silicon

  - Designed for long-term preservation

- Modern UX

  - OTA firmware updates

  - Web-based scopes and meters

  - Real-time diagnostics

Typical User Flow

- User connects PARALAX to PC via DB25.

- ESP32-TrueVGM detects LPT traffic patterns.

- UI displays: External Source Detected: OPL3LPT

- Audio is routed to speakers or real chips with effectively zero added latency.

The PC believes it is talking to a classic parallel-port sound device.

It is not.

Relationship to ESP32-TrueVGM-Chip-Player

PARALAX and ESP32-TrueVGM are independent projects that can operate separately:

- PARALAX alone

- Parallel-port audio capture

- USB audio output

- Diagnostic and monitoring tool

- ESP32-TrueVGM alone

- VGM playback

- Chip emulation

- Real-chip synthesis

Together, they form a complete, modern replacement for:

- ISA sound cards

- LPT DACs

- Ad-hoc DOS audio hacks

Designed separately. Engineered to interlock.

Philosophy

PARALAX does not reinterpret history.
It listens.

Every edge, every write, every timing quirk — preserved, transported, and rendered faithfully, whether through emulation or real silicon.

Parallel ports deserved better than bit-banged DACs.

Now they have it.

Why Not Parallel-Port Passthrough?

PARALAX intentionally does not provide a physical or electrical passthrough of the DB25 parallel port.

This is a deliberate design choice.

1. Signal Integrity Comes First

Classic parallel-port audio devices rely on:

- CPU-driven busy loops

- edge timing measured in microseconds

- undefined electrical behavior across chipsets

Introducing a passthrough would:

- add bus loading

- introduce propagation delay

- risk contention between devices

PARALAX guarantees clean, deterministic capture by being the only device on the bus.

2. There Is Nothing to “Share”

Most LPT sound devices are:

- write-only

- unidirectional

- unaware of other hardware

The PC is not querying a sound card — it is shoving bits out a port and hoping something listens.

PARALAX listens faithfully and completely.

A passthrough device would have no meaningful protocol to arbitrate.

3. Passthrough Breaks the Core Promise

PARALAX exists to:

- replace fragile DAC dongles

- eliminate ISA dependencies

- provide reproducible, clock-accurate playback

Passthrough implies:

- “this device might miss something”

- “timing might vary”

- “audio may or may not match”

That undermines the entire goal.

4. Software Passthrough Is Superior

If passthrough is needed, it can be done at a higher level, with full awareness:

- DB25 → PARALAX → ESP32

- ESP32 mirrors traffic to:

  - real sound chips

  - emulation engines

  - visualizers

  - recording tools

This allows:

- inspection

- transformation

- logging

- replay

All things impossible with a passive passthrough.

5. Future Expansion Is Not Closed

While physical passthrough is excluded, the architecture intentionally allows:

- mirrored register streams

- simultaneous real + emulated playback

- live visualization of external hardware

- conditional routing based on detected protocol

If a useful passthrough scenario emerges, it will be implemented deliberately — not electrically.

6. In Short

PARALAX does not sit between devices.

It becomes the device.

Parallel-port audio was always a hack.
PARALAX is the first time it’s been treated like a system.